# Final Project

## 4/23/2026

export GAZEBO_MODEL_PATH=~/mtre4820-final-project/install/mtre4820_rosbot/share:$GAZEBO_MODEL_PATH

echo $GAZEBO_RESOURCE_PATHD

LIBGL_ALWAYS_SOFTWARE=1 ros2 launch mtre4820_rosbot rosbot.launch.py

Images shown

ros2 run rqt_image_view rqt_image_view

Drive forward

ros2 topic pub /cmd_vel geometry_msgs/msg/Twist "{linear: {x: 0.5, y: 0.0, z: 0.0}, angular: {x: 0.0, y: 0.0, z: 0.0}}"

Spawn another robot

ros2 run gazebo_ros spawn_entity.py   -entity rosbot   -topic robot_description   -x 1.0 -y 1.0 -z .1

Teleop twist

ros2 run teleop_twist_keyboard teleop_twist_keyboard --ros-args --remap cmd_vel:=/cmd_vel

First run
source /usr/share/gazebo/setup.bash

Then run
source /opt/ros/humble/setup.bash
source ~/mtre4820_final_project/install/setup.bash

Launch
ros2 launch mtre4820_rosbot rosbot.launch.py

Make it permanent
echo "source /usr/share/gazebo/setup.bash" >> ~/.bashrc
source ~/.bashrc

## 4/30/2026

- rosbot cmd_vel

  - I am struggling to get the cmd_vel

  - When I typed in this command it hung

    - ros2 topic echo /cmd_vel

  - I tried this command

    - ros2 topic info /cmd_vel

    - Type: geometry_msgs/msg/Twist

    - Publisher count: 1

    - Subscription count: 1

  - The hz cmd hung as well

    - ros2 topic hz /cmd_vel

  - The positive and negative rotations go as such

    - To move left the vehicle must have a

      - linear velocity (0.5, 0, 0)

      - angular velocity (0, 0, 1)

    - To move right the vehicle must have a

      - linear velocity (0.5, 0, 0)

      - angular velocity (0, 0, -1)

- How to give the rosbot custom velocities

- Need to figure out theta issue and solution

- Create ROS2 node that has machine learning

  - need subscriber to rosbot camera matrix

  - need publisher of cmd_vel (geo msgs)

  - Where ML code is loaded

From your camera info:

- fx = 554.38

- width = 640

- height = 480

- cx = 320.5 (image center x)

- cy = 240.5 (image center y)

Plugging into the FOV formula:

horizontal FOV = 2 * arctan(640 / (2 * 554.38))

= 2 * arctan(0.577)

= 2 * 30°

= 60°

vertical FOV = 2 * arctan(480 / (2 * 554.38))

= 2 * arctan(0.433)

= 2 * 23.4°

= 46.8°

So your camera sees 60 degrees horizontally and 46.8 degrees vertically.

This is useful for your project because the horizontal FOV tells you the angular range across the image. So if the ambulance bounding box center is at pixel x, you can map it to a real world angle relative to the robot heading:

angle = arctan((x - cx) / fx)

Where cx = 320.5 is the image center. This gives you the direction to the ambulance which feeds into your arc calculation.

Theta idea:

- Load camera

- Use the FoV from cmarea

  - Would be good to know how to get to the camera.xml

- alpha – 450 = degree from center = beta

## 5/3/2026

Getting θ from the Image

The ambulance position in the image gives a pixel coordinate x. The angle from the robot's heading to the ambulance is:

θ = arctan((x_pixel - cx) / fx)

The focal length fx acts as a synthetic depth in pixel units, completing the right triangle needed to recover the angle even though the camera discards real depth information. The camera's 60° FOV limits θ to ±30°, which naturally bounds the arc curvature to what the robot can physically follow.

This equation is how we get the theta for the pictures.

So the full chain for data prep will be this:

- Import picture

- Import robot position and ambulance position

- Find d = cord inbetween

- Use pixel clicker on ambulance on the center

- Get x_pixel from function

- Use θ = arctan((x_pixel - cx) / fx)

- Get θ

- Locate cord midpoint

- Create perpendicular vector with d and θ

- Create length from point 3 to cord midpoint with ¼*d*sin(θ)

- Calculate R from 3-point arc

- R = |P1 - C|

- Calculate ω = v / R

- Use ω as model output in back prop

- Put this in a big for loop for the values

Reasons our model might not doing the best:

- Heavy overfitting

- Too few pictures

- Could use a model that already has edges and shape detection and freeze its weights and work onto of it

- Artifically expand the images

The model work!

Set up these libraries:

pip install opencv-python numpy pandas matplotlib tensorflow scikit-learn sympy cv_bridge

sudo apt install ros-humble-cv-bridge

Check if installed right:

python3 -c "import cv2, numpy, pandas, matplotlib, tensorflow, sklearn, sympy; print('all good')"

Two nodes from claude and chat

cladue has a working map but cannot spawn multiple ambulances

Chat has no map but can somewhat spawn with orientation for vehicle

I want the vehicle to rotate so what if we have it use a depth camera on it and if the camera sees something it does not rotate but let the ml model work

## Final doc

Our Machine learning model takes pictures of an ambulance from gazebo and gives a rosbot velocity commands for it to go to the ambulance.

### Data prep

Took pictures of ambulance and calculated their angular velocity for it. Within the simulation gazebo, we recorded the coordinates of the rosbot and the ambulance for each picture. Each picture had the ambulance in the view of the rosbot camera and that image was saved. The ambulane was put at varying position and orientations. 200 pictures were taken.

### Math part 1

Now we have to take the position of the two objects and create velocity commands from it. We are using a stock linear velocity of 0.5 m/s forward. This means we only need to calculate the angular velocity for the movement commands to be complete.

This was done by using creating a three point arc. First two are the centers of the robot and the ambulance. The third point will be a ratio using the distance between the rosbot and ambulance. The third point will be calculated like this: Go to the middle point in between the two points, draw a perpendicular line that is 1/4*distance.

Claculate a theta between the forward vector of the rosbot and the object from that vector.

The ambulance position in the image gives a pixel coordinate x. The angle from the robot's heading to the ambulance is:

θ = arctan((x_pixel - cx) / fx)

The focal length fx acts as a synthetic depth in pixel units, completing the right triangle needed to recover the angle even though the camera discards real depth information. The camera's 60° FOV limits θ to ±30°, which naturally bounds the arc curvature to what the robot can physically follow.

This equation is how we get the theta for the pictures.

Multiply the theta by the ¼*distance*sin(theta) and theta was used because as the rosbot has a ambulance more directly in front of it, the less of a bend the arc needs to be, so the theta causes the distances from the center of the cord to be smaller. The direction the line will go is dependent on whether the ambulance is on the left or right side of the image frame. Within ROS, positive angular velcotiy goes left and negative goes right.

- Center point of the circle --- P1 is the rosbot, P2 is the ambulance, and P3 is the offset mid point. Using the 3 points of the arc, we need to find the radius. So the radius can be found with two lines that intersect where the center of the circle. So we will find the center of the circle by getting the vector of V1 = P1-P2, turning into its perpendicular ray (x,y) -> (-y,x) = N1 and the point between these two at S1 = (P1+P2)/2, which we use to get its final direction D1 = S1 + N1*t. This ray will now intersect the circle, and so we do the same process for P1-P3 and find where the two rays intersect by equaling them to each other and finding the coordinates of the circle = C

- Radius of circle --- We then just need to subtract this circle center from one of our points on the circle, so it would be radius by getting the distance formula for P1-C which we will call R

- Velocity commands --- Now we know the radius, so we can calculate the angular velocity and linear velocity. The formula v/R = w (v = linear velocity and w = angular velocity) lets us find both velocities. We can have v at a set velocity forward and then w will be found once we do the calculations for R. This is how we calculate w value

### Model

We used a CNN model that had a pretrained base that locked its weights. Since we had r200 picutres, were tripled them by also adding contrast and brightness varied pictures so that totalled 600 pictures for the model to train on. The model’s output is the angular velocity for each pictures. The model was packaged and exported. We saw that not all predicted points reached the ground truth values, especially at the higher levels but that is probably from picture amount, but it was very time consuming to get the pictures because of how gazebo is set up.

## ROS2

Within ROS2 we used a rosbot and its camera. We would initialize gaebo and load the rosbot and spawn an ambulance in a random spot within its field of view. We would run a node that would import that camera data, call the ML model, and output an angular velocity. Within the video, the rover would traverse with the ambulance in the side of its view but when close to it, it would turn and move toward the vehicle head on. The node when running would constantly take in camera data, run the model, and update the angular velocity. The limiter of its corrections would be the hardware it is running on.

ML model in ros

Ros model is rosbot camera and lidar

How the node worked

The loop

## 5/24/2026

I have not used this in a while but these commands made it work:

Initializes the rosbot model:

export GAZEBO_MODEL_PATH=~/mtre4820-final-project/install/mtre4820_rosbot/share:$GAZEBO_MODEL_PATH

echo $GAZEBO_RESOURCE_PATHD

LIBGL_ALWAYS_SOFTWARE=1 ros2 launch mtre4820_rosbot rosbot.launch.py

Images shown

ros2 run rqt_image_view rqt_image_view

Drive forward

ros2 topic pub /cmd_vel geometry_msgs/msg/Twist "{linear: {x: 0.5, y: 0.0, z: 0.0}, angular: {x: 0.0, y: 0.0, z: 0.0}}"

Spawn another robot

ros2 run gazebo_ros spawn_entity.py   -entity rosbot   -topic robot_description   -x 1.0 -y 1.0 -z .1

Teleop twist

ros2 run teleop_twist_keyboard teleop_twist_keyboard --ros-args --remap cmd_vel:=/cmd_vel

First run
source /usr/share/gazebo/setup.bash

Then run
source /opt/ros/humble/setup.bash
source ~/mtre4820_final_project/install/setup.bash

Launch
ros2 launch mtre4820_rosbot rosbot.launch.py

Make it permanent
echo "source /usr/share/gazebo/setup.bash" >> ~/.bashrc
source ~/.bashrc

Work we did:

- Ambulance now is oriented correctly

- The position of the ambulance and the rosbot are being saved and the image as well so data prep is way easier.

Github

- Got github to work with ubuntu and connected with a token which we need to memorize or we cant access or push to the git hub

hub.com/Saam-Grami/ros2-rosbot-follower-machine-learning.git/': The requested URL returned error: 403

gramisaam@LAPTOP-4OTR9SNP:~/mtre4820-final-project$ git remote set-url origin https://Saam-Grami: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@github.com/Saam-Grami/ros2-rosbot-follower-machine-learning.git

git push -u origin main

How to establish git in wsl2:

cd ~/mtre4820-final-project

git init

git add .

git commit -m "initial commit"

git branch -M main

git remote add origin https://github.com/Saam-Grami/ros2-rosbot-follower-machine-learning.git

git push -u origin main

Token:

To use the git hub we needed to put our username and then a token that we generated for this workspace

git remote set-url origin https://USERNAME:TOKEN@github.com/USERNAME/repo.git

Zone identifiers delete:

find . -name "*Zone.Identifier" -delete

Update the entire git:

git add .

git commit -m "cleanup"

git push

Remove folder:

rm -rf foldername/

Remove file:

rm filename

Remote:

Check what it is set to

git remote -v

Check staged before push:

git status

Remove files form git but leave local:

git rm --cached filename

git rm --cached -r foldername/

Update local repo from github:

git pull